# (As on 10/10/99)

# ONLINE MOTORIST INFORMATION SERVICES

USER GROUPS

Users seeking online traffic information from transportation web sites can be categorized as under.

- Private Motorists

- Commuters

- Travelers

- Commercial Vehicle Operators

- Commercial Drivers

- Dispatchers

- Emergency Services

### USER NEEDS ASSESSMENT

| USERS | REQUIREMENTS/TASKS | SUB-TASKS / Information Needs |
| --- | --- | --- |
| Commuters/Private Motorists | 1.1 Incident/Congestion Details(On-route) | - Accurate location of the incident on a real-time map - Nature(viz., accident, lane closure, construction) and severity of the incident, available lanes - Estimated time to clearance - Notification of the above and alternate routes through VMS board messages(on the map also) - Report on on-going construction, bad roads |
|  | 1.2 Alternate Route Information(On-route) | 1.2.1 Time saved and Time (min, max, average) by taking a detour |
|  | - Multi-mode Travel Information(Pre-trip) | - Links to public transportation - Depict public transportation center locations on the map |
|  | - ‘Best-path’ information for point-to-point travel queries(Pre-trip) in terms of - Travel Time - Risk - Distance - Simplicity | - Maps denoting frequency of incidents on a highway - Traffic patterns over a day to help to estimate travel time or expected delay - Links to MapQuest for distance and driving direction queries. |
|  | - Information on Commonly Used Routes(Pre-trip) | - Average speed on primary routes - Travel time variation over a day |
|  | 1.6 VMS Board Locations on the map showing - Incident Details (b) Travel Times | - Flashing icons on the map for critical messages |
|  | 1.7 Miscellaneous Information | - Live traffic video to help estimate traffic volume - Links to weather conditions – fog, ice, visibility, etc. |
| ## Travelers | 1.1 Information on Commonly Used Routes(Pre-trip) | - Estimated Travel Time - Points of Interest (e.g. by giving links to AAA, gas station locations, rest areas, lodging, etc.) |
|  | 1.2 Multi-mode Travel Information(Pre-trip) | - Links to public transportation - Depict public transportation center locations on the map |
| Commercial Vehicle Operators - Commercial Drivers - Dispatchers - Emergency Services | 1.1 Incident/Congestion Information | - Nature and severity of incident - Time for clearance - Roadwork/Construction Information |
|  | 1.2 Route Information | 1.2.1 Freeway restrictions 1.2.2 Toll rate information - Express Lane Information, viz. Entry and exit points for heavy vehicles |
|  | 1.3 ‘Best-path’ in terms of safety, time, distance |  |
|  | 1.4 Dynamic Route Selection | VMS/TAR Information |

SCENARIOS

Commuters/Private Motorists:

Commuters using the ATIS for pre-trip or on-route information rely on accuracy and constant updating of information.

Pre-trip Information:

- Selecting the time of departure to a particular destination(Origin-destination query or on a common route):

(e.g.) When should I leave for Rockville (office) from College Park (home) if I have to reach at 9.00AM?

(a) Traffic patterns and/or estimated travel times over a day

- Deciding on the quickest path to a common/essential place:

(e.g.) Which route will take me sooner to Baltimore from College Park at 2000 hrs.?

- Estimated travel time at the time of day

(b) Incident-related delay/abnormal conditions such as roadwork, congestions

- Selecting multiple modes of transport or alternatives:

Links to public transportation web sites

On-route Information:

- Deciding on alternate routes:

- Real-time maps with live cameras in the vicinity of the incident to help the user estimate congestion in different routes.

- Speed radar data

Travelers:

Pre-trip Information:

(i) 	Selecting scenic places off a route:

(e.g.) What are the scenic spots within 5 minutes driving distance off the freeway on the way from Hyattsville to Greenbelt?

- Locate points-of-interest off the route on the map

- Provide links to AAA, etc.

- Estimating travel time to a destination:

(e.g.) How long will it take to reach Shenandoah from College Park if I start at 1300 hrs. on a Saturday?

- Estimated travel time at the time of day

(b) Incident-related delay or abnormal conditions

Commercial Vehicles:

(i) 	Choosing express lanes as alternate routes:

(e.g.) The mainline highway is blocked because of a football match; which express lanes can I use and what are the entry and exit points for heavy vehicles?

- Express lane heavy-vehicle and HOV restrictions information

- Entry/Exit point locations for express lanes or other alternate routes.

REFERENCE:

- Human Factors in Intelligent Transportation Systems: Woodrow Barfield, Thomas A Dingus