IBM 360 vs B5000

|  | IBM 360 (1964) | Burrows 5000 (1961) |
| --- | --- | --- |
| Instruction size(s) | Variable (16, 32 and 48 bits) | 12 bit (syllables) |
| Character Size | 4 bits (Binary coded decimal) and 8bit (Eight bit coded decimal) ASCII | 6 bits (8 characters/48 bits word) |
| Integer Size | 32 bits | 48 bits (no floating point/integer conversion. Integer’s exponent field is 0) |
| Floating Point Size | 32 bits and 64 bits | 48 bits |
| Address Size | 24 bits (ignore 8 most significant bits in 32 bits) | Program Reference Table with 1024 entries (Segmented) |

- B5000

  - Stack architecture

  - High level language computer (ALGOL & COBOL)

  - Java Virtual Machine uses stack-oriented short operands

  - HP calculators use Reverse Polish Notation

- Legacy of IBM 360

  - 8 bit characters

  - 32 bit size

  - ascii characters

  - general purpose register model (base + index register addressing)

  - floating point registers separate from integer registers

  - 2’s complement

  - binary compatibility

  - segmented virtual memory

  - asynchronous I/O

  - byte addressability

- IBM 370

  - Paged virtual memory

RISC vs CISC

- On the desktop, CISC won.

- Embedded systems where power and every nickel counts, RISC won.

- RISC architectures got more complicated over time. Remained load/store architectures for the most part with complicated register operations.

- RISC pushes the burden onto compilers to do things efficiently.

- Code size is not necessarily better with RISC architecture.

- RISC moves complexity from hardware to software. Software => best effort (can patch stuff later). Hardware => fix everything (culture to get it write).

- RISC architecture not difficult to clone. IBM 360, on the other hand, very complex and difficult to clone.