- Worse options than SMT

  - Multiprocessors, putting more cores on the chip vs multithreading

  - Superscalar

  - Better than both because they had problems with both vertical and horizontal waste. SMT avoided both, didn’t waste cycles behind long instructions and was able to execute more parallel instructions.

- Vertical waste – pipelining stall

- Horizontal waste – functional units are wasted (limits to ILP)

- Extra HW resources that SMT requires:

  - Expand register file to keep track of all threads

  - Double memory bandwidth

- How does SMT differ from multithreading?

  - SMT

    - issue from more than one thread in the same cycle.

    - Get multiple instructions from multiple threads in the same clock cycle.

  - Multithreading - picks one thread to issue from in each cycle.

- What unit is the bottleneck for SMT?

  - SMT is limited by instruction fetch.

- 2 threads from which they fetch the instructions, 8 instructions per thread per clock cycle.

- Priority scheme – Icount.

  - dependant on how many instructions/thread were executing.

  - Higher priority if fewer instructions are present.

- Had to increase pipeline depth to have 2 stages for reading and 2 stages for writing. Downside for stretching out pipeline is 1.5% impact.

- Assumptions made about computer

  - 8 instr fetch/decode per clk cycle

  - 6 integer units (4 can do ld/store)

  - 4 fp units

  - more renaming registers (comparable to power5)

  - 12 instr/clk cycle retire

  - 16MB cache

  - …see paper for more and compare to ILP competition slide

- When would design evolve? 3 years.

- Performance measurements:

  - Speedup

  - Instructions per clock

  - Decide they’re done when the first thread stops.

- Competitors and Performance comparisons to SMT

  - Superscalar (coarse & fine-grained)

  - Multiprocessors with 2 and 4 cores

  - For the same number of threads, 2x better performance vs superscalar and fine-grained multithreading, 1.5x vs multiprocessors

- Comparison to Wall’s ILP claims

  - Wall claimed that even with optimistic model (infinite number of functional units), there are limits to ILP.

  - SMT claims very aggressive compared to state of microprocessors today.

  - Included resource contention in their model, given 6 integer + 4 fp units. 4 loads and stores at once is pretty aggressive.

  - Looked at 8-way multithreading threading…addressed “given resource-rich environment, how good is SMT?”

- Intel did simple SMT model but got very limited results. IBM put lots more resources and got 1.2x speedup. Both companies looked at the technology and said 2-way threading is most reasonable.