- 12 functional units (Itanium has 11 or 12)

- 80 mHz clock rate…so fast because it’s designed to be very small for its era. C shaped to minimize wire length.

- Register state was around 4 or 5 kbytes

- Memory size was 8Mb. 1 million words (word-oriented machine)…8byte words

- Latency roughly same as 2GHz processor.

- 4 most striking features of CRAY

  - memory speed

  - 4 types of chips used (2 types of NAND gates, 16x4 register file, 1kbit SRAM memory chip)

  - cooling system

  - speed

- Instruction set architecture

  - Load/store

  - 3 registers

- No virtual memory…used base & limit registers for relocation and protection

- Floating point format different

  - truncation

  - Didn’t do full multiply arrays

  - Reciprocal instead of divide (faster than divide)

- Vector vs scalar speed

  - Minimum size vector 2 or 3 times faster than scalar

  - 10x faster

  - world’s fastest scalar computer

- 138-200 gigaflops…Pentium-4/AMD 64 are around 4-5 gigaflops