- Motivation:

  - Authors worried about performance

  - Smaller disks rather than larger ones

  - Reliability of smaller disks was a concern

- Validity of predictions?

  - Processor performance didn’t double every year

  - Disk capacity more than 30% per year

- Perf. Figures of merit to compare RAID: read modify write

  - Not a realistic way of evaluating the system

- Group sizes 25 bytes…pretty big and unrealistic

- RAID2 (ECC) has a lower MTTF than RAID3(parity) because more disks in system

- Use higher RAID number if more I/O operations (pick based on access patterns/workload)

  - HP AutoRAID automatically administers the system and selects RAID level based on workload.

- Contribution: use commodity hardware to build bigger/better system

- Got wrong: tradeoff between efficiency in redundant data vs inefficient but higher reliability

- Synchronization isn’t a big deal…didn’t buy much

- RAID in HW vs RAID in SW

  - HW => Get it right the first time…works well

  - SW => fixable, can be patched so not as good quality

- MTTF suggested by manufacturer is different from MTTF in the field due to:

  - power failures, air condition failures, vibrations etc

  - Human Operator can grab wrong disk due to bad interface.